7449/1

DRAWING AND DESIGN (THEORY)

Paper 1

Time 2½ hours

BUNAMFAM

MARKING SCHEME

Kenya Certificate of Secondary Education

DRAWING AND DESIGN

MARKING SCHEME

SECTION A (50 marks) Answer ALL the questions in this section in the spaces provided.

1.	Explain technical drawing as means of communication.	(3 marks)

i)	is a discipline that is able to be understood world wide by those who have studied it. It is made

possible due to the standardization of conventions, abbreviation and symbols.

ii)  NB: Technical drawing is a means of communication between engineers and / or designers and the

production / manufacturing industries.	3 marks

2.	Name three types of lines and specify the pencil grade to be used in each case.	(3  marks)

i)	Thick continuous - visible outlines B or HB

ii)  Medium Thin Cont - Dimensional or external outlines.

-	2 H or H

iii) Very thin cont. lines - construction or guide lines - 3H or 4H.	3 marks

3.	a)  Illustrate how an AO drawing paper can be sub-divided to generate paper sizes A1, A2 and

A3.	(3 marks)

(420×594)

(841 × 594)

(297×594)

b)  Show dimensions of

A0 = 841 × 1189 = 841 × 1189

A1 = 841 × 1189 = 841 × 594

2

A2 = 841 × 594 = 420 × 594

2

A3  = 594 × 420 = 297 × 420

4.	Use standard symbols and abbreviations to represent each of the following:

i)   Cylinder                                 ii)   Machined surface.	(4 marks)

5.	Construct a triangle ABC with angles in the ratio 3 : 4 : 6 given the length of the base as 50mm. (6 marks)

Solution

3 + 4 + 6 = 13

Draw semi-circle and divide it into 13 parts

Single-out the ratios 3 : 4 : 6 on the semi-circle.

6.	Construct a diagonal scale of 2 : 1 having an accuracy of 0.1mm to read to a maximum of

40mm. Show a reading of 28.3mm	(6 marks)

Solution

Scale length

= R.F × MAX. LENGTH

= 2 × 40mm = 80mm

1

7.	Locus of point C for one revolution of crank OB	(6 marks)

LOCUS OF POINT C

8.	Using a ruler and pair of compasses only, construct a REGULAR HEPTAGON. Whose sides are 30mm long.	(6 marks)

HEPTAGON SIDES 30mm

9.	                                                      (6 marks)

TRUNCATED TRIANGULAR PYRAMID

10. Define  the following properties of materials:-

i)   Hardness - when it can withstand scratching, wear or Abrasion, indentation by harder bodies. e.g.

marking knife, files etc.

ii)  Toughness  - the ability of a material to withstand impact load or hammering load.

iii) Elasticity - Ability of a material to deform under load and return to its original shape or size when the load is removed. So long as it does not exceed its elastic limit.

iv) Plasticity - ability of a material to deform under load and retain its new shape when the load is removed. e.g. soft steel.	(4 marks)

SECTION B (20 marks) This question is compulsory. Candidates are advised to spend not more than one hour on this question.

11. Figure 7 shows parts of a retort stand clamp drawn in first angle projection.  Assemble the parts and draw FULL SIZE, the following views of the vice in third angle projection:

a)  A sectional front elevation along the cutting plane P - P

b)  End elevation in the direction of arrow X

c)  Plan

Insert three leading dimensions and do not show hidden details.

| PARTS NAME | MATL | NO. OFF |
| --- | --- | --- |
| E PIN | M.S | 1 |
| D SCREW | M.S | 1 |
| C LOWER JAW | M.S | 1 |
| B UPPER JAW | M.S | 1 |
| A SCREWED ROD | M.S | 1 |
| SCALE | 1 : 1 | NAME |

SECTION C (30 marks)

12. Figure 8 shows the mouth of a cup having Ø45mm and a handle protruding 10 mm.

If the cup is rolled on the surface AA for one complete revolution, construction the locus of point X on the handle.	(15 marks)

SOLUTION

13. Figure 9 shows a square pyramid transacted along X - X and Y - Y.

Copy the given front elevation, complete the plan and draw the end elevation in the direction of arrow U.	(15 marks)

14. Make an isometric drawing from  the two views given in FIGURE.     (15 marks)